Enterprise Network Implementation & Firewall Architecture
UniFi Dream Machine Pro Centralized Routing & Cisco L2 Switch Trunking

| Primary ISP: PLDT Fiber (WAN 1) | Secondary ISP: Converge ICT (WAN 2) | SIP Trunk: PLDT Direct to Yeastar PABX |
| --- | --- | --- |
| Core Switch: Cisco WS-C3850-12XS (Pure L2) | Access Switches: 3x Cisco 3850-48PoE+ (Pure L2) | Document Version: 2.0 (UDM-Pro Gateway/DHCP Only) |

1. Network Topology & Port Mapping

This document details the step-by-step setup where UDM Pro handles all Layer 3 IP Gateways, Inter-VLAN Routing, Firewall enforcement, and DHCP server services. The Cisco 3850 XS Core and Access switches operate strictly as Layer 2 devices performing 802.1Q VLAN trunking and access port assignments.

| Source Device & Port | Destination Device & Port | Connection Type / Speed | Configured Role / Allowed VLANs |
| --- | --- | --- | --- |
| UDM Pro — SFP+ Port 11 | Cisco 3850 XS Core — SFP+ Port 1 | 10G SFP+ Optical / DAC | 802.1Q Trunk / All VLANs (10,20,30,40,50,60,70,90) |
| Cisco 3850 XS Core — Port 3 | Server 2 Access Switch — SFP Port 1 | 1G SFP Module / Cable | 802.1Q Trunk / All VLANs (10,20,30,40,50,60,70,80,90) |
| Cisco 3850 XS Core — Port 4 | Server 3 Access Switch — SFP Port 1 | 1G SFP Module / Cable | 802.1Q Trunk / All VLANs (10,20,30,40,50,60,70,80,90) |
| Cisco 3850 XS Core — Port 5 | Server 4 Access Switch — SFP Port 1 | 1G SFP Module / Cable | 802.1Q Trunk / All VLANs (10,20,30,40,50,60,70,80,90) |
| Access Switches (Server 2/3/4) | Cross-Switch Links (SFP 2, 3, 4) | 1G SFP Interconnects | 802.1Q Trunk / Daisy-Chain Inter-Switch Links |
| PLDT Fiber Modem | Yeastar PABX — WAN/LAN Port | Direct RJ45 Ethernet | Dedicated SIP Trunk Voice Connection |
| Yeastar PABX — LAN Port | Access Switch Port | 1GbE RJ45 Patch Cable | Access Port — VLAN 10 (Phone Network) |

2. Logical Subnet & DHCP Configuration Scheme (Managed on UDM Pro)

All SVIs, Gateway IPs, and DHCP servers are created and served directly by the UDM Pro.

| VLAN ID | Name / Category | Gateway IP (UDM Pro) | Subnet Mask | DHCP Scope / Server Notes |
| --- | --- | --- | --- | --- |
| VLAN 10 | Servers | 10.0.10.1 | 255.255.255.0 (/24) | 10.0.10.10 - 10.0.10.254 (SERVERS, Cisco Switchtes) |
| VLAN 20 | Operations | 10.0.20.1 | 255.255.255.0 (/24) | 10.0.20.10 - 10.0.20.254 (Operations PC) |
| VLAN 30 | Accounting | 10.0.30.1 | 255.255.255.0 (/24) | 10.0.30.10 - 10.0.30.254 (Accounting PCs) |
| VLAN 40 | Budget | 10.0.40.1 | 255.255.255.0 (/24) | 10.0.40.10 - 10.0.40.254 (Budget PC) |
| VLAN 50 | COA | 10.0.50.1 | 255.255.255.0 (/24) | 10.0.50.10 - 10.0.50.254 (COA PC) |
| VLAN 60 | Phone | 10.0.60.1 | 255.255.255.0 (/24) | 10.0.60.10 - 10.0.60.254 (VOIP, Phones) |
| VLAN 70 | Access Point | 10.0.70.1 | 255.255.255.0 (/24) | 10.0.70.10 - 10.0.70.254 (Access Points) |
| VLAN 80 | CCTV | 10.0.70.1 | 255.255.255.0 (/24) | 10.0.70.10 - 10.0.70.254 (CCTV) |
| VLAN 90 | AV | 10.0.90.1 | 255.255.255.0 (/24) | 10.0.90.100 - 10.0.90.254 AV |

3. Centralized Routing Implementation Steps

Architecture Note Because IP routing is disabled on the Cisco Core switch (no ip routing), the UDM Pro acts as the sole router, default gateway, DHCP server, and firewall for all subnets.

Step 1: Configure Networks & DHCP on UDM Pro

Perform the following in the UniFi Network Console (Settings > Networks):

1. Click Add New Network.

2. Set Name (e.g., VLAN 10 - Phone).

3. Set Router to UDM Pro.

4. Set VLAN ID (e.g., 10).

5. Under Gateway IP/Subnet, enter 10.10.10.1/24.

6. Enable DHCP Server and set pool range (e.g., 10.10.10.10 - 10.10.10.254).

7. Repeat these steps for VLANs 20, 30, 40, 50, 60, 70, and 90.

Step 2: Configure Cisco 3850 XS Core Switch (Pure Layer 2 Trunking)

Apply these IOS commands on the Core switch to disable L3 routing and pass all VLANs to the UDM Pro:

! Ensure IP Routing is Disabled on Cisco Core Switch no ip routing ! Define VLAN Database vlan 10,20,30,40,50,60,70,80,90 exit ! Uplink Port to UDM Pro (SFP+ Port 1 -> UDM Pro Port 11) interface TenGigabitEthernet2/0/1 description UPLINK-TO-UDM-PRO-PORT11 switchport mode trunk switchport trunk allowed vlan 10,20,30,40,50,60,70,80,90 no shutdown ! Downlink Ports to Access Switches (SFP Ports 3, 4, 5) interface range TenGigabitEthernet2/0/3 - 5 description DOWNLINK-TO-ACCESS-SWITCHES switchport mode trunk switchport trunk allowed vlan 10,20,30,40,50,60,70,80,90 no shutdown

Step 3: Configure Cisco 3850 Access Switches (Server 2, 3, 4)

Apply these IOS commands on each access switch:

! Access Switch Configuration no ip routing vlan 10,20,30,40,50,60,70,80,90 exit ! Uplink to Core Switch (SFP Port 1) interface GigabitEthernet1/1/1 description UPLINK-TO-CORE-SWITCH switchport mode trunk switchport trunk allowed vlan 10,20,30,40,50,60,70,80,90 no shutdown ! Cascade / Inter-Switch Links (SFP Ports 2, 3, 4) interface range GigabitEthernet1/0/2 - 4 description CASCADE-LINK-TO-ACCESS-SWITCH switchport mode trunk switchport trunk allowed vlan 10,20,30,40,50,60,70,80,90 no shutdown

4. Inter-VLAN Firewall Rules Specification (Configured on UDM Pro)

Since UDM Pro handles all routing, inter-VLAN firewall rules are enforced directly in UniFi Network under Settings > Security > Firewall Rules > LAN IN.

Firewall Rule Evaluation Order Rules are processed top-down. Explicit ALLOW rules (1001–1006) are evaluated first. Rule 2000 drops all unapproved cross-VLAN traffic, securing each department.

| Rule # | Rule Name & Purpose | Source Subnet / IP | Destination Subnet / IP | Protocol & Ports | Action |
| --- | --- | --- | --- | --- | --- |
| 1001 | Allow Established & Related Traffic | Any | Any | State: Established, Related | ACCEPT |
| 1002 | Allow Workstations to Yeastar PABX | VLAN 30, 40, 50 | 10.0.60.10/32 (PABX IP) | TCP 80, 443, 5060, UDP 5060, 10000-20000 | ACCEPT |
| 1003 | Allow Workstations to Servers | VLAN 30, 40, 50 | VLAN 10 (10.0.70.0/24) | TCP 80, 443, 445 (SMB), 3389 (RDP), 1433 (SQL) | ACCEPT |
| 1004 | Allow AP Management to Controller | VLAN 20 (10.10.50.0/24) | 10.0.70.5/32 (UniFi Controller) | UDP 3478, TCP 8080, 8443 | ACCEPT |
| 1005 | Allow Operations to CCTV NVR | VLAN 20 (Operations) | 10.0.80.5/32 (CCTV NVR) | TCP 80, 443, 554 (RTSP), 8000 | ACCEPT |
| 1006 | Allow DNS / DHCP to Gateways | RFC1918 Private Subnets | UDM Pro Gateways (10.10.x.1) | UDP 53 (DNS), UDP 67/68 (DHCP) | ACCEPT |
| 2000 | Block All Inter-VLAN Traffic | RFC1918 (10.0.0.0/8, 172.16.0.0/12, 192.168.0.0/16) | RFC1918 Private Subnets | ANY Protocol / Ports | DROP |

5. Cisco Access Switch Port Provisioning

A. Desktop PC / Standard Workstation Ports (VLAN 20, 30, 40, 70)

! Example: Accounting PC on Access Switch Port 10 interface GigabitEthernet1/0/10 description WORKSTATION-ACCOUNTING switchport mode access switchport access vlan 30 spanning-tree portfast bpduguard enable no shutdown

B. IP Phone with Daisy-Chained Workstation (VLAN 10 Voice + VLAN 20 Data)

! Example: IP Phone + Budget PC on Access Switch Port 11 interface GigabitEthernet1/0/11 description IP-PHONE-AND-PC switchport mode access switchport access vlan 20 ! Data VLAN (Budget) switchport voice vlan 10 ! Voice VLAN (Phone) spanning-tree portfast bpduguard enable no shutdown

C. UniFi Access Points (VLAN 50 Native Management + Multi-SSID Trunks)

! Example: UniFi Access Point on Access Switch Port 12 interface GigabitEthernet1/0/12 description UNIFI-ACCESS-POINT switchport trunk encapsulation dot1q switchport mode trunk switchport trunk native vlan 50 ! Native Management VLAN switchport trunk allowed vlan 10,20,30,40,50,60,70,80,90 spanning-tree portfast trunk no shutdown

6. Testing, Verification & Troubleshooting

| Test Target | CLI / Verification Command | Expected Result |
| --- | --- | --- |
| Verify Cisco Core Routing Status | show ip route | Displays % IP routing not enabled (Confirms L2 mode). |
| Verify VLAN Database | show vlan brief | All VLANs (10, 20, 30, 40, 50, 60, 70, 90) active on switch. |
| Verify Inter-Switch Trunks | show interfaces trunk | Port 1/0/1 and downlinks show status trunking with VLANs allowed. |
| Verify Client DHCP Assignment | Connect client PC to Port 10 | Receives IP address from UDM Pro (e.g., 10.10.30.x). |
| Verify Inter-VLAN Blocking | ping 10.10.40.10 from Accounting PC | Timed out / Dropped by UDM Pro Firewall Rule 2000. |